Checklist for Ergonomic Risk Factors

| Things to look for: | Possible solutions, depending on further analysis: |
| --- | --- |
| Prolonged hunched or elevated shoulder while holding the phone | - Telephone headset - Speakerphone |
| Elbows splayed out (shoulder abduction) | - Lower work surface - Lower chair armrests - Bring chair armrests in closer - Awareness and habit training |
| Raised or tensed shoulders | - Habit or tension training - Lower worksurface or keyboard - Lower chair armrests - Raise chair, if foot contact with the floor can be maintained |
| Twisting the head to the side | Bring viewed item closer to centerline of view |
| Elbow flexed for long periods using the telephone | - Telephone headset - Speakerphone |
| Elbow or forearm resting for long periods on hard or sharp worksurface, chair armrests | - Pad or round surfaces, corners, and armrests - Replace armrests - Telephone headset - Habit training |
| Wrists bent to the sides when using side keys | - Habit training - Keyboard with more accessible keys or split keyboard design |
| Wrists bent back (extended) or forward (flexed) for prolonged periods | - Habit training - Palm rest - Lower, raise, or change slope of the keyboard |
| Wrists or palms resting for long periods on hard or sharp keyboard or work surfaces | - Habit training - Palm rest - Padded or rounded surfaces, corners |
| Hands held actively over the keyboard during keying pauses | - Habit training - Palm or forearm rest |
| Rapid, sustained, or prolonged keying | - Greater work variety - Aggressive break schedule - Reduce overtime |
| Forceful keying, key pounding | - Habit training - Light-touch keyboard |
| Significant amounts of hand stapling, punching, lifting, opening mail, or other forceful exertions, especially combined with awkward postures | - Mechanical aids, such as electric stapler or punch - Reduce size of lifted loads - Bring heavy loads close to the body, at a medium height - Substitute sliding (worksurface) or wheeling (floor) - Sharpen letter openers |
| Prolonged mouse use | - Greater work variety - Aggressive break schedule - Alternate hands - Alternative pointing devices - Arm support, including small table - Mouse close to body (extended keyboard tray) - Learn keystroke substitutes for menus |
| Prolonged sitting, especially in only one posture | - Greater work variety - Aggressive break schedule - Chair that supports posture change, through movement, size, or easy adjustability - Habit training - Move phone and printer to the other side of the office to force standing, or suggest standing when on phone - Check chair fit - Monitor in-out mechanism - Sit-stand worksurface |
| Lumbar back area not supported | - Lumbar cushion - Backrest height and tilt - Check chair fit, especially backrest/lumbar height |
| Feet dangling, not well supported, or a posture which seems to put pressure on the backs of the thighs | - Lower chair - Lower worksurface - Habit training - Foot rest (last resort) |
| Chair backrest not used for long periods | - Check chair fit, especially seat pan depth and height - Check leg room - Check monitor distance and character height - Habit training |
| Twisted torso | - Rearrange work - Provide more knee space - U-shaped worksurface layout - Swivel chair |
| Frequent or prolonged leaning or reaching | - Rearrange work - Mouse pad, palm or forearm rest - Bring mouse and keyboard closer to body |
| Working with one or both arms "reaching" toward a mouse or keyboard | - Bring keyboard closer to body - Mouse pad, palm or forearm rest - Bring mouse closer to keyboard |
| Light sources that can be seen by the worker | - Cover or shield light sources - Rearrange work arena - Lower other viewed objects to lower field of view |
| Reflected glare on the screen | - Shield light sources - Shade screen - Glare screen - Move monitor so light enter from side angle, not back Do NOT tip monitor down - Lower light levels - Move light sources |
| Too much contrast between screen and surroundings or document; worker feels relief when bright areas are shielded | - Lower ambient light levels - Turn off, reposition, or dim task lights - Block offending light sources - Change screen polarity to black on white |
| Very bright ambient lighting (above 500 lux or 50 fc) or shadowed areas caused by over-illumination | Lower ambient light levels to 200-500 lux (20-50 fc) and use task lights |
| Monitor closer than approximately 65 cm (25") | - Push monitor back (enlarge font size) - Habit training for reclining - Computer glasses - Bring keyboard forward, possibly with a keyboard tray |
| Different viewed objects (screen, documents) at different distances from the eyes | Use document stand or otherwise equalize distances to within about 10 cm (4") if rapid viewing changes are required |
| Screen or documents not oriented perpendicular to the line of sight (tipped back slightly is even better) | Change monitor, document stand angle |
| Prolonged near focusing throughout the day with few far-focusing opportunities | - Move monitor back as far as possible - Habit training - Rearrange space to provide view - Introduce glazing |
| Monitor image dim, fuzzy, flickery, small, or otherwise difficult to read | Upgrade monitor Use software to enlarge image |
| Shiny, low-contrast, or small-print documents | Improve lighting on documents if documents cannot be changed |
| Forward head posture (peering) or squinting | - Lower monitor - Tilt monitor back - Check for monitor image quality problems, character height or monitor distance - Suggest consultation with vision specialist |
| Eyestrain complaints | - Check all aspects of visual environment - Suggest consultation with vision specialist |
| Neck extended backwards, head tilted back, even slightly | - Remove CPU from under monitor - Remove tilt-swivel base from monitor (leave ventilation space) - Check for bifocals and suggest full-frame "computer glasses" prescription |
| Neck severely flexed (downward) | - Tilt face of monitor back - Tilt document - do not lay flat on work surface - Raise document or monitor to a comfortable height - Adjust posture - Habit retraining - Check glasses for proper prescription |