Workstation Assessment Checklist

To be Filled out by Ergonomics Assessor

Employee Name: ________________________	Division\Bureau: _______________________________

Employee’s Position Title: _________________________  Time in Position: ________________________

Employee’s Supervisor/Office Manager: _____________________________________________________

Work Site Location (Building/Floor/Suite): ___________________________________________________

Ergonomic Coordinator: __________________________________________________________________

This checklist is designed to help you adapt/adjust am employee’s computer workstation and video display terminal (VDT) to help prevent common stresses and injuries associated with VDT usage.  This should be used in addition to the pre-assessment ergonomics questionnaire the employee needs to complete prior to the assessment.

Check the following:

Chair - A well designed and adjusted chair improves circulation and posture and can prevent back strain and fatigue. Does the chair incorporate the following features?

|  | Chair height and backrest tension/angle are easily adjustable. |
| --- | --- |
|  | Chair is equipped with a padded seat and back cushion. |
|  | Front edge of seat pan is rounded and seamless. |
|  | Seat width and depth accommodate specific employee (seatpan not too big/small).. |
|  | Seat front does not press against the back of knees and legs. |
|  | Chair has non-slippery upholstery with porous “breathable” fabric |
|  | Backrest provides support for employee's lower back (lumbar area). |
|  | Backrest is at least 18” tall x 14” wide |
|  | Seat tilts back only slightly as the backrest tilts back |
|  | Chair swivels easily on casters |
|  | Chair is supported with a five-leg pedestal base for stability |
|  | Pedestal base of chair is at least 24”around |
|  | Height and width adjustable armrests are provided if needed |

Comments: ____________________________________________________________________________

### Posture

|  | Sit up straight |
| --- | --- |
|  | Sit back in the chair with feet flat on the floor |

Comments: ____________________________________________________________________________

Foot/ Leg Positions - The proper foot and leg positions help prevent stress in your neck and shoulders plus prevent cramping and stiffness in your legs.

|  | Feet are flat on the floor or on a footrest |
| --- | --- |
|  | 3” – 6” of leg room between legs and workstation |
|  | Thighs are parallel to floor |

Comments: ____________________________________________________________________________

Keyboard and Mouse / Arm and Wrist Positions - Proper positioning of your keyboard and your arms and wrists will improve comfort and avoid injury.

|  | Determine how keyboard and Mouse is used i.e.: light, moderate, heavy. |
| --- | --- |
|  | Keyboard height is adjustable. |
|  | Keyboard is horizontal or negatively sloped at about 0-15 degrees |
|  | Keyboard is prevented from slipping |
|  | Wrists are relaxed and straight (neutral) |
|  | Wrist rest or parallel support armrests are padded and free of sharp edge |
|  | Arms are close to body with elbows at 90-degree angle |
|  | Wrist/ forearms are straight, parallel to floor and do not rest on sharp edges. |
|  | Mouse is at same level as keyboard and within easy reach |
|  | Keyboard tray (if used) is stable and large enough to hold keyboard and mouse |
|  | Mouse fits the hand of the employee |

Comments: ____________________________________________________________________________

Workstation - A well-designed and arranged workstation encourages good posture and provides less stress on the body.

|  | Workstation/table height is adjustable if needed. Height of workstation: |
| --- | --- |
|  | Table surface is deep enough to allow adequate room for workstation components |
|  | Adequate space to adjust height/ location of monitor |
|  | Front edge of table is rounded |
|  | Materials used most often are arranged within easy reach |
|  | Document holder the same height and distance as the display screen |
|  | Storage drawers located under desk/table do not restrict knee clearance |

Comments: ____________________________________________________________________________

Display Screen/ Monitor - Adjusting the viewing distance, height, angle, and clarity of the display screen can reduce eye and neck strain.

|  | Monitor is directly in front of employee |
| --- | --- |
|  | Display screen is at an arm’s length away from your eyes |
|  | Top line of display (print) is slightly below eye level |
|  | Monitor is adjusted for employee with bifocals and trifocals (lower than usual) |
|  | Display is tilted slightly to reduce reflections and glare |
|  | Display screen is clean and free of flickering |
|  | Brightness and contrast controls are adjusted for viewing comfort |

Comments: ____________________________________________________________________________

Lighting/ Glare Reduction - Proper lighting and glare reduction will reduce the strain on your eyes.

|  | Indirect or shielded lighting is used to reduce reflections and glare |
| --- | --- |
|  | Window blinds or drapes are adjusted or closed when needed |
|  | Lighting levels are adjusted (where possible) throughout the day |
|  | Face of display screen is at right (90 degree) angles to windows |
|  | Anti-glare screen/ filter is used if necessary |
|  | Task lighting or desk lamp is adjusted to avoid glare and reflections |

Comments: ____________________________________________________________________________

Working Conditions

|  | Tasks are varied throughout the day with micro breaks and stretches |
| --- | --- |
|  | Workstation and equipment are adjustable and well maintained |

Comments: ____________________________________________________________________________

### Telephone use

|  | Determine how telephone is used i.e.: light, moderate, heavy. |
| --- | --- |
|  | Utilize speakerphone or headset if necessary. |

Comments: ____________________________________________________________________________

You may benefit from the following suggestions and activities:

- When using your telephone, hold the phone in your hand to prevent neck and shoulder stress or use a headset.

- Pay attention to your posture. Avoid awkward positions. When sitting, use the backrest of the chair.

- Strive to adjust your workstation so that knees, hips, and elbows are at minimum a 90-degree angle.

- Raise or lower your chair according to the workstation height while assuring your feet still rest flat on the floor or a foot rest.

- Use a lighter touch at the keyboard to reduce excessive force.

- Keep those items on your desk that you use most often within easy reach, such as your mouse, keyboard, telephone, and calculator.

- Vary your job activities throughout the day if possible.

- Don’t stay in one position for an extended time. Get up, move around, do some stretching.

- Give the eyes a rest throughout the day. Focus at an object at a distance, then near, at distance, etc.

- Periodically do stretching exercises at your workstation and while on break. Take shorter but more frequent breaks from your work.